APB Subsystem Verification Test Plan

- Introduction

This document presents the verification plan for the AMBA APB-based subsystem integrating GPIO, UART, and SPI peripherals. The verification environment is developed using SystemVerilog and UVM methodology to ensure functional correctness, protocol compliance, and reliable communication between the APB master and peripheral slaves.

- Design overview

The AMBA APB subsystem design consists of one APB master controller, an APB interconnect module, and three APB slave peripherals: UART, SPI, and GPIO. The APB master initiates read and write transactions, while the interconnect module performs address decoding and routes transactions to the appropriate peripheral based on the address map. Each peripheral is designed as an APB-compliant slave capable of handling 32-bit data transactions.

RTL module names:

- apb_uart.sv

- apb_gpio.sv

- apb_spi.sv

- apb_master.sv

- apb_interconnect.sv

- apb_top.sv

- Verification scope

The verification scope of the APB subsystem includes functional verification of the APB master, APB interconnect, and APB slave peripherals including UART, SPI, and GPIO modules.

The verification environment is designed to verify the following functionalities:

- APB protocol compliance for read and write transactions.

- Address decoding and peripheral selection in the APB interconnect.

- Correct data transfer between APB master and slave peripherals.

- GPIO register read/write operations and direction control.

- UART transmit and receive functionality.

- SPI transmit and receive functionality.

- Valid and invalid address accesses.

- PSLVERR generation for invalid transactions.

- PREADY response handling.

- Reset behavior and default register initialization.

The verification environment uses SystemVerilog and UVM methodology with reusable components such as drivers, monitors, sequencers, scoreboards, and coverage collectors to ensure complete functional verification of the subsystem.

- Feature list

The design features are listed below.

1. APB protocol compliant read and write transactions

2. The design has 32-bit address and 32-bit data transfer support

3. APB master transaction generation

4. Address decoding through APB interconnect

5. Peripheral slave selection based on address mapping

6. GPIO input, output, and direction control registers

7. UART transmit functionality (serial transmission)

8. UART receive functionality (serial transmission)

9. SPI transmit functionality (serial transmission)

10. SPI receive functionality (serial transmission)

11. APB ready signal handling (PREADY)

12. APB slave error response generation (PSLVERR)

13. Valid and invalid address access handling

14. Reset and register initialization behavior

15. Register read/write access verification

16. Concurrent peripheral access through APB interconnect

17. Functional coverage collection for APB transactions and peripheral operations

- Verification strategy

The verification environment for the APB subsystem is developed using SystemVerilog and UVM methodology. A reusable and modular UVM-based testbench architecture is implemented to verify the functionality of the APB master, APB interconnect, and APB slave peripherals including UART, SPI, and GPIO.

The verification strategy includes constrained-random and directed testing approaches to verify both normal and corner-case scenarios. The APB master agent is responsible for generating APB read and write transactions to different peripheral slaves based on the address map.

The UVM environment consists of the following components:

• Sequencer for transaction generation

• Driver for driving APB protocol and slave signals (handshaking mechanisms) to the DUT

• Monitor for capturing APB bus activity and slave outputs

• Scoreboard for data checking and protocol validation

• Functional coverage collectors for coverage analysis

• Assertions for APB protocol checking

The verification environment verifies:

• Correct APB protocol behavior

• Address decoding functionality

• Peripheral register access operations

• UART transmit and receive operations

• SPI transmit and receive operations

• GPIO read/write and direction control functionality

• Error handling for invalid accesses

• Reset behavior and default register values

Functional coverage and assertions are used to measure verification completeness and protocol compliance.

- Test Cases

| Testcase ID | Testcase Name | Description |
| --- | --- | --- |
| TC_01 | apb_reset_test | Verify reset behavior of APB subsystem and all peripherals |
| TC_02 | apb_write_gpio_test | Verify APB write operation to GPIO registers |
| TC_03 | apb_read_gpio_test | Verify APB read operation from GPIO registers |
| TC_04 | apb_write_uart_test | Verify APB write to UART TX register |
| TC_05 | apb_read_uart_test | Verify APB read from UART RX/STATUS registers |
| TC_06 | apb_write_spi_test | Verify APB write operation to SPI registers |
| TC_07 | apb_read_spi_test | Verify APB read operation from SPI registers |
| TC_08 | uart_tx_test | Verify UART serial transmission |
| TC_09 | uart_rx_test | Verify UART serial reception |
| TC_10 | spi_transfer_test | Verify SPI MOSI/MISO transfer |
| TC_11 | gpio_direction_test | Verify GPIO direction control |
| TC_12 | invalid_address_test | Verify PSLVERR for invalid addresses |
| TC_13 | back_to_back_write_test | Verify continuous APB write transactions |
| TC_14 | back_to_back_read_test | Verify continuous APB read transactions |
| TC_15 | mixed_read_write_test | Verify mixed read/write accesses across slaves |
| TC_16 | random_apb_write_test | Random APB writes to all peripherals |
| TC_17 | random_apb_read_test | Random APB reads from all peripherals |
| TC_18 | random_apb_rw_test | Fully randomized APB read/write traffic |
| TC_19 | uart_spi_gpio_mix_test | Random accesses across UART/SPI/GPIO |
| TC_20 | stress_test | Long random regression with multiple transactions |

- Assertion Plan

7.1 Objectives of Assertions

The main objectives of assertions are:

- Verify APB protocol timing

- Detect illegal signal behavior

- Ensure correct APB state transitions

- Validate slave response timing

- Detect invalid address accesses

- Ensure proper UART/SPI/GPIO operation

- Improve debug efficiency during simulation

7.2 APB Protocol Assertions

APB Setup Phase Check

Objective: Verify that PENABLE remains LOW during setup phase.

Assertion:

property apb_setup_phase;

@(posedge PCLK)

PSEL && !PENABLE |-> ##1 PENABLE;

endproperty

assert property(apb_setup_phase);

Expected Behavior

- PSEL asserted first

- PENABLE asserted in next cycle

APB Access Phase Check

Objective: Verify that transfer completes only when PREADY is HIGH.

Assertion:

property apb_access_phase;

@(posedge PCLK)

PENABLE |-> PSEL;

endproperty

assert property(apb_access_phase);

Expected Behavior

- PENABLE cannot assert without PSEL

APB Stable Address Check

Objective: Ensure address remains stable during APB transfer.

Assertion:

property apb_addr_stable;

@(posedge PCLK)

PSEL && PENABLE |-> $stable(PADDR);

endproperty

assert property(apb_addr_stable);

Expected Behavior

- PADDR must not change during access phase

APB Stable Write Data Check

Objective: Ensure write data remains stable during write transfer.

Assertion:

property apb_wdata_stable;

@(posedge PCLK)

PSEL && PENABLE && PWRITE |-> $stable(PWDATA);

endproperty

assert property(apb_wdata_stable);

Expected Behavior

- PWDATA must remain constant during write

Invalid Address Error Check

Objective: Verify PSLVERR assertion for invalid addresses.

Assertion:

property invalid_addr_check;

@(posedge PCLK)

(PSEL && PENABLE &&

!(PADDR inside {

32'h0000_0000,

32'h0000_0004,

32'h0000_0008,

32'h0001_0000,

32'h0001_0004,

32'h0001_0008,

32'h0002_0000,

32'h0002_0004,

32'h0002_0008,

32'h0002_000C

})) |-> PSLVERR;

endproperty

assert property(invalid_addr_check);

Expected Behavior

- Invalid addresses must assert PSLVERR

7.3 UART Assertions

UART Start Bit Check

Objective: Verify UART transmission starts with start bit.

Assertion:

property uart_start_bit;

@(posedge PCLK)

tx_state == START |-> tx == 1'b0;

endproperty

assert property(uart_start_bit);

Expected Behavior

- UART TX line goes LOW during start bit

UART Stop Bit Check

Objective

Verify UART transmission ends with stop bit.

Assertion

property uart_stop_bit;

@(posedge PCLK)

tx_state == STOP |-> tx == 1'b1;

endproperty

assert property(uart_stop_bit);

Expected Behavior

- UART TX line returns HIGH during stop bit

7.4 SPI Assertions

SPI Chip Select Check

Objective

Verify chip select assertion during SPI transfer.

Assertion

property spi_cs_assert;

@(posedge PCLK)

spi_state == TRANSFER |-> cs == 1'b0;

endproperty

assert property(spi_cs_assert);

Expected Behavior

- cs must remain LOW during transfer

SPI Clock Activity Check

Objective

Verify SPI clock toggles during transfer.

Assertion

property spi_clk_toggle;

@(posedge PCLK)

spi_state == TRANSFER |-> $changed(sclk);

endproperty

assert property(spi_clk_toggle);

Expected Behavior

- sclk toggles during SPI communication

7.5 GPIO Assertions

GPIO Output Stability Check

Objective

Verify GPIO outputs remain stable without APB write.

Assertion

property gpio_stable;

@(posedge PCLK)

!(PSEL && PENABLE && PWRITE &&

PADDR == 32'h0000_0000)

|-> $stable(gpio_out);

endproperty

assert property(gpio_stable);

Expected Behavior

- GPIO outputs change only during valid writes

7.6 Assertion Coverage Goals

| Assertion Type | Goal |
| --- | --- |
| APB Protocol Assertions | 100% |
| UART Assertions | 100% |
| SPI Assertions | 100% |
| GPIO Assertions | 100% |
| Error Handling Assertions | 100% |

- Coverage Plan

8.1 Coverage Objectives

The main objectives of coverage collection are:

- Measure APB protocol exercise

- Verify all peripherals are accessed

- Ensure all register addresses are covered

- Verify read/write transactions

- Measure UART/SPI/GPIO functionality coverage

- Ensure error scenarios are exercised

- Identify untested corner cases

- Track verification progress

8.2 Coverage Types

The verification environment includes:

| Coverage Type | Description |
| --- | --- |
| Functional Coverage | Measures feature-level verification |
| Code Coverage | Measures RTL execution coverage |
| Assertion Coverage | Measures assertion activity |
| FSM Coverage | Measures state transition coverage |
| Toggle Coverage | Measures signal toggle activity |

8.3 APB Functional Coverage

APB Address Coverage

Objective: Ensure all valid slave addresses are accessed.

Coverpoints

coverpoint PADDR
{
    bins gpio_addr[] =
    {
        32'h0000_0000,
        32'h0000_0004,
        32'h0000_0008
    };

    bins uart_addr[] =
    {
        32'h0001_0000,
        32'h0001_0004,
        32'h0001_0008
    };

    bins spi_addr[] =
    {
        32'h0002_0000,
        32'h0002_0004,
        32'h0002_0008,
        32'h0002_000C
    };
}

Coverage Goal

- 100% address coverage

APB Read/Write Coverage

Objective: Ensure both read and write transactions occur.

Coverpoints

coverpoint PWRITE
{
    bins READ  = {0};
    bins WRITE = {1};
}

Coverage Goal

- 100% read/write coverage

APB Slave Selection Coverage

Objective

Verify access to all peripherals.

Coverpoints

coverpoint PADDR[31:16]
{
    bins GPIO = {16'h0000};
    bins UART = {16'h0001};
    bins SPI  = {16'h0002};
}

Coverage Goal

- All slaves accessed

APB Error Coverage

Objective: Verify invalid address accesses.

Coverpoints

coverpoint PSLVERR
{
    bins ERROR    = {1};
    bins NO_ERROR = {0};
}

Coverage Goal

- At least one invalid access detected

8.4 UART Coverage

UART TX Data Coverage

Objective: Verify UART transmission activity.

Coverpoints

coverpoint tx_reg[7:0]
{
    bins low_range  = {[8'h00:8'h3F]};
    bins mid_range  = {[8'h40:8'h7F]};
    bins high_range = {[8'h80:8'hFF]};
}

Coverage Goal

- Full byte range coverage

UART State Coverage

Objective: Verify all UART FSM states are exercised.

Coverpoints

coverpoint tx_state
{
    bins IDLE_STATE  = {IDLE};
    bins START_STATE = {START};
    bins DATA_STATE  = {DATA};
    bins STOP_STATE  = {STOP};
}

Coverage Goal

- All UART states covered

8.5 SPI Coverage

SPI Transfer Coverage

Objective: Verify SPI transfer operation.

Coverpoints

coverpoint spi_state
{
    bins IDLE_STATE     = {IDLE};
    bins START_STATE    = {START};
    bins TRANSFER_STATE = {TRANSFER};
    bins DONE_STATE     = {DONE};
}

Coverage Goal

- All SPI states covered

SPI Control Register Coverage

Objective: Verify SPI control operations.

Coverpoints

coverpoint control_reg[0]
{
    bins START_TRANSFER = {1};
}

Coverage Goal

- SPI transfer initiated successfully

8.6 GPIO Coverage

GPIO Output Coverage

Objective: Verify GPIO output patterns.

Coverpoints

coverpoint gpio_out
{
    bins zero      = {32'h00000000};
    bins all_ones  = {32'hFFFFFFFF};
    bins patterns[] =
    {
        32'hA5A5A5A5,
        32'h5A5A5A5A
    };
}

Coverage Goal

- Representative GPIO patterns covered

GPIO Direction Coverage

Objective: Verify GPIO direction control.

Coverpoints

coverpoint gpio_dir
{
    bins input_mode  = {32'h00000000};
    bins output_mode = {32'hFFFFFFFF};
}

Coverage Goal

- Input and output modes exercised

8.7 Cross Coverage

APB Address vs Read/Write

Objective: Ensure every slave address is accessed using both read and write operations.

Cross Coverage

cross PADDR, PWRITE;

Coverage Goal

- All valid addresses covered with both reads and writes

UART State vs TX Busy

Objective: Verify UART busy behavior across FSM states.

Cross Coverage

cross tx_state, status_reg[0];

8.8 Coverage Goals Summary

| Coverage Type | Goal |
| --- | --- |
| Functional Coverage | ≥ 95% |
| Code Coverage | ≥ 90% |
| FSM Coverage | 100% |
| Toggle Coverage | ≥ 90% |
| Assertion Coverage | 100% |
| Cross Coverage | ≥ 90% |

8.9 Coverage Closure Strategy

Coverage closure will be achieved by:

- Directed testcases

- Randomized APB transactions

- Corner-case testing

- Error injection testing

- Protocol stress testing

- Cross-coverage analysis

- Additional constrained-random scenarios

- Corner Cases

9.1. APB Protocol Corner Case

| Corner Case ID | Description | Expected Behavior |
| --- | --- | --- |
| CC_APB_01 | Access invalid address | PSLVERR asserted |
| CC_APB_02 | Read during write transaction | Transaction handled correctly |
| CC_APB_03 | Write during active transfer | No protocol violation |
| CC_APB_04 | Back-to-back transactions | No data corruption |
| CC_APB_05 | PSEL deasserted during transfer | Transaction aborted safely |
| CC_APB_06 | PENABLE asserted without PSEL | Ignored by slave |
| CC_APB_07 | Reset during transaction | Bus returns to IDLE safely |
| CC_APB_08 | Random address accesses | Correct slave decoding |
| CC_APB_09 | Continuous read/write operations | Stable operation |
| CC_APB_10 | Simultaneous invalid control combinations | No unknown behavior |

9.2 GPIO Coner Case

| Corner Case ID | Description | Expected Behavior |
| --- | --- | --- |
| CC_GPIO_01 | Write all zeros to GPIO_OUT | Outputs cleared |
| CC_GPIO_02 | Write all ones to GPIO_OUT | Outputs set |
| CC_GPIO_03 | Random GPIO patterns | Correct output update |
| CC_GPIO_04 | Change GPIO direction dynamically | Direction updates correctly |
| CC_GPIO_05 | Read GPIO input while direction changes | Stable read data |
| CC_GPIO_06 | Read from invalid GPIO address | PSLVERR asserted |
| CC_GPIO_07 | Reset during GPIO operation | Registers cleared |
| CC_GPIO_08 | Consecutive GPIO writes | Latest value retained |

9.3 UART Coner Case

| Corner Case ID | Description | Expected Behavior |
| --- | --- | --- |
| CC_UART_01 | Transmit 0x00 | Correct UART frame |
| CC_UART_02 | Transmit 0xFF | Correct UART frame |
| CC_UART_03 | Continuous UART transmissions | No frame corruption |
| CC_UART_04 | Start new transmission while busy | Ignored or flagged |
| CC_UART_05 | Reset during UART transmission | TX returns idle |
| CC_UART_06 | Invalid UART register access | PSLVERR asserted |
| CC_UART_07 | Verify stop bit generation | Stop bit = 1 |
| CC_UART_08 | Random UART payloads | Correct serial output |
| CC_UART_09 | RX start-bit glitch | Ignored safely |
| CC_UART_10 | Multiple 32-bit APB writes | Data transmitted as 4 UART frames |

9.4 SPI Corner Case

| Corner Case ID | Description | Expected Behavior |
| --- | --- | --- |
| CC_SPI_01 | SPI transfer with all zeros | Correct MOSI output |
| CC_SPI_02 | SPI transfer with all ones | Correct MOSI output |
| CC_SPI_03 | Random SPI payload | Correct serial shifting |
| CC_SPI_04 | Reset during SPI transfer | SPI returns IDLE |
| CC_SPI_05 | Multiple back-to-back SPI transfers | No data corruption |
| CC_SPI_06 | Invalid SPI address access | PSLVERR asserted |
| CC_SPI_07 | SPI start command while busy | Ignored safely |
| CC_SPI_08 | Verify CS assertion/deassertion | Correct chip-select behavior |
| CC_SPI_09 | Verify SCLK toggling | Clock toggles correctly |
| CC_SPI_10 | Random MISO input patterns | Correct RX capture |

- Regression Plan

| Regression Type | Description |
| --- | --- |
| Smoke Regression | Basic sanity tests |
| Feature Regression | Peripheral feature verification |
| Protocol Regression | APB protocol compliance |
| Random Regression | Constrained random transactions |
| Error Regression | Invalid access and error tests |
| Stress Regression | Long continuous traffic |
| Corner Regression | Boundary and abnormal scenarios |
| Reset Regression | Reset recovery tests |

- Signoff Criteria

| Coverage Type | Target |
| --- | --- |
| APB transaction coverage | 100% |
| GPIO coverage | >95% |
| UART coverage | >95% |
| SPI coverage | >95% |
| Address coverage | 100% |
| Read/write coverage | 100% |
| Error coverage | 100% |
| FSM state coverage | 100% |